Q1. What is Sensor Calibration & Why is it Important?

​Sensor calibration is the process of comparing a sensor's raw output measurements against a known, accurate standard (reference point) to identify deviations and eliminate errors.

​Why it is important in IoT:

​Manufacturing Variations: Two identical sensors off the same factory line will often give slightly different analog readings under the exact same conditions.

​Environmental Degradation: Sensors degrade over time due to wear, temperature spikes, or chemical exposure (especially soil probes), shifting their raw baseline values.

​Contextual Meaning: A raw analog reading like 612 means nothing to an end-user or an automation rule; it needs to be mapped to a meaningful, human-readable value like 45% moisture.

​2. Calibrating an Analog Soil Moisture Sensor

​An analog soil moisture sensor typically measures volumetric water content through electrical resistance or capacitance, outputting a value from 0 to 1023 (on Arduino's 10-bit ADC).

​Because air offers max resistance and pure water offers minimum resistance, your raw data points often run opposite to reality (e.g., higher numbers mean drier soil). Calibration standardizes this into an accurate percentage scale (0\% to 100\%).

3. The Two-Point Calibration Method

​The Two-Point Calibration method maps the sensor’s raw output linearly across two trusted reference extremes: the lowest possible threshold (Dry) and the highest possible threshold (Wet).

​Step-by-Step Procedure:

​Find the Dry Point (R_{\text{dry}}): Keep the sensor completely dry in the open air. Record the stable raw analog value printed to your Serial Monitor. Let's say this value is 1010. This represents 0\% Moisture.

​Find the Wet Point (R_{\text{wet}}): Submerge the sensor up to its maximum boundary line in a cup of pure water. Record the stable raw value. Let's say this value is 230. This represents 100\% Moisture.

​The Linear Formula: Use these two points to calculate the true percentage for any unknown incoming reading (R_{\text{raw}}). In Arduino, you implement this math simply using the built-in map() function: