Q explain the setup() and loop() function in Arduino.

Ans. The Anatomy of an Arduino Sketch

​Arduino programs are built on a specific structure that dictates the lifecycle of your code.

​1. setup() Function

​The setup() function is the first thing that runs when your Arduino board is powered on or reset.

​Execution: It runs exactly once.

​Purpose: It is used to initialize the hardware environment. This typically includes:

​Setting pin modes (e.g., pinMode(13, OUTPUT);).

​Starting serial communication (e.g., Serial.begin(9600);).

​Initializing variables or libraries.

​2. loop() Function

​After the setup() function has completed, the Arduino enters the loop() function.

​Execution: It executes the code inside its curly braces sequentially, and once it reaches the end, it automatically restarts from the top.

​Purpose: This is where your main logic lives. Because it runs indefinitely, it is responsible for actively monitoring sensors, making decisions, and controlling actuators.

Q what  happens if you put a long delay() inside loop()?

Ans.  When you place a long delay() inside the loop() function, you essentially "pause" the entire execution of your Arduino's microcontroller. Because the Arduino is a single-threaded device, it can only do one thing at a time.

​The Consequences of a Blocking delay()

​Processor "Freezing": When the code hits delay(n), the processor stops everything—it stops checking sensors, stops responding to button presses, and stops updating outputs. It effectively sits in an idle loop doing nothing for the specified duration.

​Missed Events: If you have a sensor that needs to be checked constantly (like an emergency stop button or a high-frequency motion sensor), the Arduino will be "blind" to those inputs while it is in the middle of a delay().

​Cumulative Latency: If you have multiple delay() calls in a loop, their durations add up. For example, if you have three delay(1000) calls in your loop, the total time for one full cycle becomes at least 3 seconds. The system's responsiveness decreases linearly with every millisecond added to the delay.

Q How does this affect sensor reading responsiveness? What is the non-blocking alternative?

Ans. When you place a long delay() inside the loop(), it severely impacts the sensor reading responsiveness by making your Arduino "deaf" to any new information during the delay period.

​Impact on Sensor Responsiveness

​Blocking Execution: The delay() function stops the processor from executing any instructions for the specified time. If your delay(1000) is active, the Arduino spends 1000ms doing absolutely nothing.

​Latency in Data Acquisition: If a sensor changes its value (e.g., a motion sensor is triggered) while the Arduino is "sleeping" during a delay(), the Arduino will not detect that change until the delay has finished.

​Missed Events: In fast-paced applications, such as catching a short button press or detecting a quick pulse from an encoder, the event might happen and end entirely within the delay() period. The Arduino will never know it happened, leading to missed data or failed input detection.

​The Non-Blocking Alternative: millis()

​The non-blocking approach involves using the millis() function, which acts as an internal, free-running timer that tracks how many milliseconds have passed since the Arduino was powered on.

​Instead of pausing the entire program, you use a logic structure that checks if enough time has elapsed to perform an action, while letting the loop() continue to run freely.

​How it works:

​Record the time an event last happened.

​Continuously check in every loop() iteration if current_time - last_time >= interval.

​Execute the code only if that condition is true.