1. Explanation of I2C Protocol in IoT

​I2C (Inter-Integrated Circuit) is a synchronous, multi-master, multi-slave, packet-switched serial communication bus protocol.

​In IoT systems, microcontrollers frequently need to gather data from multiple peripheral sensors. Instead of using separate dedicated pins for every single sensor (which quickly runs out of hardware pins), I2C allows a master device (like an Arduino or ESP32) to connect to up to 127 slave devices (sensors, displays, or EEPROMs) using just two shared wires.

​2. What are SDA and SCL Lines?

​I2C relies strictly on two lines to manage all communication:

​SDA (Serial Data Line): The bidirectional line over which data bits are transferred between the master and the slaves.

​SCL (Serial Clock Line): The unidirectional clock signal line generated entirely by the master device. It synchronizes the timing of data transmission bit-by-bit across the bus.

​3. How I2C Addressing Works

​Since dozens of sensors share the exact same SDA and SCL wires, the master needs a way to target a specific device. This is handled via unique physical hardware addresses:

​Unique Bit Addresses: Every I2C-compatible sensor has a hardcoded 7-bit unique address assigned by the manufacturer (e.g., an OLED display might sit at 0x3C, while a gyro sensor sits at 0x68).

​The Broadcast: When the master initiates communication, it transmits a "Start Condition" followed immediately by the 7-bit address of the specific sensor it wants to talk to, plus an 8th bit indicating whether it wants to Read or Write data.

​The Acknowledge (ACK): Every sensor on the bus hears the address. The sensor whose address matches the broadcast pulls the SDA line down to send an Acknowledge (ACK) bit back to the master. All other sensors recognize it isn't their address and safely ignore the rest of the transmission.

​4. 3 Common IoT Sensors that use I2C

​When designing IoT projects, you will routinely find these common components relying on the I2C interface:

| Sensor model | Sensor type | Typical 12c hex address |
| --- | --- | --- |
| MPU6050 | 6 axis gyroscope and accelerometer | 0*68 or 0*69 |
| BME280 | Barometric pressure,temp and humidity sensor | 0*76 or 0*77 |
| BH1750 | Digital ambient light intensity sensor | 0*24 or 0*5c |