SUMMER SCHOOL ’26

INTERNSHIP REPORT

FRESHGUARD: IoT-BASED FOOD FRESHNESS MONITORING AND PREDICTION SYSTEM

Submitted in partial fulfilment of the requirements of the Summer School ’26 Internship Programme

Submitted by:
Khushkaranpreet kaur

IoT-41
Department of  Computer  science and Engineering
Batch 2024-2028
Punjabi university patiala

Internship Organization:
Indian Institute of Technology Jammu

Internship Duration:
10 June 2026  to  20 July 2026

Academic Session:
2025–2026

INTERNSHIP COMPLETION CERTIFICATE

Attach the Internship Completion Certificate issued by IIT Jammu here.

STUDENT DECLARATION

I hereby declare that the internship report entitled "FreshGuard: IoT-Based Food Freshness Monitoring and Prediction System" is an original record of the training and project work carried out during the Summer School '26 Internship Programme at the Indian Institute of Technology Jammu from 10 June 2026 to 20 July 2026.

The internship involved structured training in the field of Internet of Things (IoT), embedded systems, sensors, microcontrollers, cloud-based communication, artificial intelligence, and intelligent monitoring systems. The programme provided practical exposure to designing and developing IoT-enabled solutions capable of solving real-world engineering problems.

As part of the internship, I worked with my teammates Ridhika Bansal, Sayan Biswas, and Tanish Kumar Aggarwal to design and develop FreshGuard, an IoT-enabled food freshness monitoring and prediction system. Every team member contributed throughout the project lifecycle, including problem identification, literature review, system design, component selection, hardware development, embedded programming, AI/ML model integration, cloud communication, testing, documentation, and final presentation.

The project was completed under the guidance of Majid Majeed, Programme Coordinator, Summer School '26, Indian Institute of Technology Jammu.

I further declare that the work presented in this report is original and has not been submitted previously for the award of any degree, diploma, or certificate.

Date: 2 August 2026
Place: Jammu

Digital Signature of Student

Khushkaranpreet kaur

IoT-41

ACKNOWLEDGEMENT

I would like to express my sincere gratitude to the Indian Institute of Technology Jammu for providing me with the opportunity to participate in the Summer School '26 Internship Programme. The internship offered an excellent platform to strengthen my theoretical understanding while gaining valuable practical experience in Internet of Things technologies, embedded systems, artificial intelligence, cloud computing, and smart sensing applications.

I am deeply thankful to Majid Majeed, Programme Coordinator, for his continuous guidance, encouragement, and technical support throughout the internship. His valuable suggestions and mentoring played a significant role in helping us understand the complete development process of an IoT-enabled intelligent monitoring system, from conceptualization and system design to implementation, testing, and documentation.

I would also like to express my gratitude to my teammates Ridhika Bansal, Sayan Biswas, and Tanish Kumar Aggarwal, with whom I collaboratively developed FreshGuard: IoT-Based Food Freshness Monitoring and Prediction System. The successful completion of this project was made possible through our collective efforts in research, planning, hardware integration, embedded programming, AI/ML implementation, cloud connectivity, testing, troubleshooting, documentation, and presentation. Working together enhanced not only our technical knowledge but also our teamwork, communication, and project management skills.

I would like to thank all the instructors, mentors, laboratory staff, and fellow participants of the Summer School for sharing their knowledge, providing valuable feedback, and creating a collaborative learning environment throughout the internship.

Finally, I express my heartfelt gratitude to my family and friends for their constant encouragement, motivation, and unwavering support throughout the internship and during the preparation of this report.

ABSTRACT

The increasing demand for food safety and quality monitoring has created a need for intelligent systems capable of detecting food spoilage before it becomes harmful for consumption. Conventional methods of determining food freshness are often subjective, time-consuming, and unsuitable for continuous monitoring. The integration of the Internet of Things (IoT), cloud computing, and Artificial Intelligence (AI) offers an effective solution by enabling real-time monitoring, data analysis, and predictive decision-making.

The Summer School '26 Internship Programme at the Indian Institute of Technology Jammu, conducted from 10 June 2026 to 20 July 2026, provided extensive theoretical and practical training in Internet of Things technologies, embedded systems, sensor interfacing, cloud communication, and intelligent system development. The programme enabled participants to apply these concepts to solve real-world engineering challenges through project-based learning.

During the internship, our team developed FreshGuard: IoT-Based Food Freshness Monitoring and Prediction System, an intelligent IoT solution designed to monitor food storage conditions and predict freshness using environmental sensing and machine learning techniques. The system continuously measures temperature, humidity, gas concentration, and ambient light intensity using the DHT11, MQ-2, and LDR sensors connected to an ESP32 microcontroller. The collected sensor data is processed locally and transmitted to the ThingSpeak cloud platform for real-time visualization and storage.

To enhance decision-making, an AI/ML-based prediction model analyzes the collected environmental parameters and estimates the freshness level of stored food. Based on predefined thresholds and prediction results, the system displays the freshness status on an OLED display while activating an audible buzzer whenever abnormal conditions are detected. Furthermore, ThingSpeak integrated with IFTTT generates automated email and SMS alerts, allowing users to take preventive action before food spoilage occurs.

The project involved system requirement analysis, hardware selection, embedded programming using Arduino IDE, cloud integration, AI model implementation, testing, troubleshooting, and performance evaluation. The final outcome was a functional prototype capable of providing continuous food freshness monitoring, intelligent prediction, cloud-based visualization, and remote notification, demonstrating the practical application of IoT and Artificial Intelligence in smart food quality management.

The internship significantly strengthened technical knowledge in IoT architecture, embedded programming, cloud computing, AI integration, hardware-software interfacing, system testing, teamwork, technical documentation, and engineering problem-solving while providing valuable experience in developing an industry-oriented smart monitoring solution.

CHAPTER 1

INTRODUCTION

1.1 Background

The rapid advancement of the Internet of Things (IoT) has transformed the way physical devices interact with the digital world. IoT enables sensors, embedded processors, communication technologies, and cloud platforms to work together for collecting, processing, and exchanging real-time information. These intelligent systems have found widespread applications in healthcare, agriculture, industrial automation, environmental monitoring, transportation, smart homes, and food safety.

Food spoilage is one of the major global challenges affecting public health, food security, and economic sustainability. According to various international food organizations, a significant percentage of food produced worldwide is lost due to improper storage, delayed detection of spoilage, and inadequate monitoring of environmental conditions. Traditional methods of determining food freshness mainly rely on visual inspection, smell, texture, or expiry dates. These methods are often subjective, inaccurate, and incapable of providing continuous monitoring during storage or transportation.

Perishable food products such as fruits, vegetables, dairy products, meat, and seafood undergo chemical and biological changes over time. During decomposition, microorganisms produce gases such as methane, ammonia, hydrogen, and other volatile compounds. Simultaneously, environmental factors including temperature, humidity, and light exposure influence the rate of spoilage. Continuous monitoring of these parameters can provide valuable information regarding the freshness and safety of stored food.

The Internet of Things offers an effective solution by integrating sensors, embedded controllers, cloud platforms, and intelligent algorithms into a single monitoring system. IoT-based food monitoring systems can continuously collect environmental data, analyze changing conditions, generate early warnings, and assist users in making informed decisions regarding food quality. Such systems not only improve food safety but also reduce unnecessary food wastage and contribute to sustainable resource utilization.

Recent developments in Artificial Intelligence (AI) and Machine Learning (ML) have further enhanced the capabilities of IoT systems. Instead of merely displaying sensor readings, AI models can analyze historical and real-time data to predict the freshness level of food and estimate the likelihood of spoilage. This predictive capability enables preventive action before food quality deteriorates significantly.

During the Summer School '26 Internship Programme conducted at the Indian Institute of Technology Jammu, participants received structured training in Internet of Things technologies, embedded systems, sensors, microcontrollers, communication protocols, cloud computing, and intelligent system development. The programme emphasized practical learning through project-based implementation of real-world engineering problems.

As part of the internship, our team developed FreshGuard: IoT-Based Food Freshness Monitoring and Prediction System. The project combines IoT technology with Artificial Intelligence to continuously monitor food storage conditions using environmental sensors. The system measures temperature, humidity, gas concentration, and ambient light intensity through multiple sensors connected to an ESP32 microcontroller. Sensor readings are transmitted to the ThingSpeak cloud platform for remote monitoring, while an AI/ML model predicts food freshness based on collected environmental data. Whenever abnormal conditions indicating possible spoilage are detected, the system immediately activates a buzzer and sends email or SMS notifications through IFTTT, allowing users to take timely preventive action.

FreshGuard demonstrates how intelligent sensing, cloud connectivity, and predictive analytics can be integrated into a practical IoT solution that enhances food quality management, minimizes food waste, and promotes safer food storage practices.

1.2 Purpose of the Internship

The primary purpose of the Summer School '26 Internship Programme was to provide students with practical exposure to Internet of Things technologies and intelligent embedded system development. The programme was designed to bridge the gap between theoretical classroom learning and practical engineering implementation by allowing participants to work on real-world problems using modern IoT technologies.

The internship aimed to familiarize students with sensors, actuators, microcontrollers, embedded programming, wireless communication, cloud platforms, and system integration techniques required for designing connected smart devices. Participants also gained experience in hardware assembly, software development, testing, debugging, and technical documentation.

Another important objective of the internship was to encourage collaborative learning through team-based project development. Students were expected to identify an engineering problem, conduct technical research, design an appropriate solution, develop a functional prototype, evaluate system performance, and present their work in a professional manner.

For this purpose, our team selected the problem of food freshness monitoring and developed FreshGuard, an IoT-enabled intelligent monitoring system capable of continuously observing environmental conditions affecting food quality. The project integrated embedded systems, cloud computing, and Artificial Intelligence to provide real-time monitoring and predictive freshness assessment.

The internship also strengthened analytical thinking, engineering design skills, teamwork, technical communication, and problem-solving abilities through hands-on implementation and continuous interaction with faculty mentors.

1.3 Internship Objectives

The main objectives of the internship were:

- To understand the architecture and working principles of the Internet of Things.

- To study various sensors, actuators, microcontrollers, and embedded systems.

- To learn embedded programming using Arduino IDE.

- To understand sensor interfacing techniques with ESP32 microcontrollers.

- To study cloud-based IoT communication using ThingSpeak.

- To understand wireless communication protocols used in IoT applications.

- To develop practical skills in circuit design and hardware assembly.

- To integrate multiple sensors into a single intelligent monitoring system.

- To understand data acquisition, processing, storage, and visualization.

- To apply Artificial Intelligence and Machine Learning techniques for predictive analysis.

- To develop an IoT-based solution for food freshness monitoring.

- To generate automated alerts using cloud services and IFTTT.

- To test, validate, and troubleshoot an integrated hardware-software system.

- To improve teamwork, communication, documentation, and presentation skills.

1.4 Scope of Work

The scope of the internship included both theoretical learning and practical implementation of Internet of Things technologies. Throughout the programme, participants studied the complete lifecycle of IoT system development, from identifying a real-world problem to developing and evaluating a working prototype.

The major areas covered during the internship included:

- Introduction to the Internet of Things

- IoT architecture and applications

- Sensors and actuators

- Embedded systems

- ESP32 microcontroller programming

- Analog and digital sensor interfacing

- Wireless communication

- Cloud computing

- ThingSpeak platform

- Data acquisition and visualization

- Artificial Intelligence and Machine Learning fundamentals

- Circuit design and hardware integration

- System testing and troubleshooting

- Team-based project development

- Technical documentation and presentation

The practical component of the internship focused on the design and development of FreshGuard, an IoT-enabled food freshness monitoring system. The project was intended for educational and demonstration purposes and showcased how embedded sensing, cloud connectivity, and AI-based prediction techniques can be combined to develop an intelligent food quality monitoring solution.

The developed prototype continuously measures environmental conditions affecting food freshness, predicts spoilage using machine learning techniques, uploads sensor data to the cloud, displays live information on an OLED screen, and generates automated notifications through email and SMS whenever abnormal conditions are detected.

1.5 Internship Duration

The internship was conducted at the Indian Institute of Technology Jammu from 10 June 2026 to 20 July 2026.

The total duration of the internship was approximately six weeks.

1.6 Organization of the Report

This report has been organized into ten chapters to provide a systematic presentation of the internship activities and project work.

- Chapter 1 introduces the internship, its objectives, scope, duration, and the motivation behind the FreshGuard project.

- Chapter 2 presents the internship overview, organizational details, responsibilities, expected deliverables, and weekly timeline.

- Chapter 3 describes the training and learning phase, including IoT fundamentals, sensors, embedded programming, communication technologies, cloud computing, and AI concepts covered during the internship.

- Chapter 4 provides a detailed explanation of the FreshGuard project, including the problem statement, objectives, hardware architecture, system workflow, AI/ML freshness prediction methodology, cloud integration, hardware implementation, testing, and final prototype.

- Chapter 5 presents the weekly work log and summarizes the activities completed throughout the internship.

- Chapter 6 discusses the technical and practical challenges encountered during project development along with the solutions implemented.

- Chapter 7 highlights the technical, professional, and interpersonal skills developed during the internship.

- Chapter 8 summarizes the overall learning outcomes and knowledge gained through practical implementation.

- Chapter 9 presents a self-assessment of strengths, areas for improvement, and reflections on the internship experience.

- Chapter 10 concludes the report by summarizing the overall internship experience, project achievements, future enhancements, and career aspirations.

CHAPTER 2

INTERNSHIP OVERVIEW

2.1 Internship Details

| Particular | Details |
| --- | --- |
| Student Name | Khushkaranpreet kaur |
| Department | Computer science and Engineering |
| Batch | 2024 |
| College | Punjabi university patiala |
| Internship Programme | Summer School '26 |
| Internship Domain | Internet of Things (IoT) and Drone building |
| Organization | Indian Institute of Technology Jammu |
| Internship Location | Jammu |
| Duration | 10 June 2026 – 20 July 2026 |
| Programme Coordinator | Majid Majeed |
| Major Project | FreshGuard: IoT-Based Food Freshness Monitoring and Prediction System |
| Team Members | Khushkaranpreet Kaur, Sayan Biswas, Ridhika Bansal, Tanish Kumar Aggarwal |
| Nature of Work | IoT Training, AI/ML Integration, Embedded System Design and Development |
| Technologies Used | ESP32, DHT11, MQ-2, LDR, ThingSpeak, Arduino IDE, IFTTT, AI/ML |
| Final Project Outcome | Functional IoT Prototype for Food Freshness Monitoring and Prediction |

2.2 Internship Objectives

The Summer School '26 Internship Programme was designed to provide students with practical exposure to modern Internet of Things technologies and their real-world applications. The programme emphasized experiential learning by combining theoretical instruction with hands-on laboratory sessions and collaborative project development.

The major objectives of the internship were:

- To understand the architecture and components of IoT systems.

- To gain practical knowledge of embedded systems and sensor interfacing.

- To develop programming skills for ESP32-based applications using Arduino IDE.

- To understand cloud-based IoT communication using the ThingSpeak platform.

- To learn data acquisition, storage, and visualization techniques.

- To explore Artificial Intelligence and Machine Learning for predictive analytics.

- To design and develop an intelligent IoT-based engineering solution.

- To strengthen teamwork, communication, and technical documentation skills.

- To understand the complete product development lifecycle, from requirement analysis to prototype validation.

These objectives were achieved through the successful development of FreshGuard, an intelligent IoT-enabled food freshness monitoring system capable of sensing environmental conditions, predicting freshness using AI/ML, and notifying users through cloud-based alerts.

2.3 Roles and Responsibilities

Throughout the internship, I actively participated in both the training programme and the development of the team project. My responsibilities included technical implementation, collaboration with teammates, testing, documentation, and presentation.

The major responsibilities undertaken during the internship included:

- Attending theoretical and practical IoT training sessions.

- Studying IoT architecture and real-world applications.

- Learning embedded programming using Arduino IDE.

- Understanding the working principles of ESP32 microcontrollers.

- Studying sensors for temperature, humidity, gas concentration, and light intensity measurement.

- Participating in project selection and requirement analysis.

- Conducting literature review on food spoilage monitoring techniques.

- Designing the overall system architecture for FreshGuard.

- Integrating DHT11, MQ-2, and LDR sensors with ESP32.

- Developing embedded programs for sensor data acquisition.

- Uploading real-time sensor data to the ThingSpeak cloud platform.

- Implementing AI/ML-based freshness prediction.

- Configuring IFTTT for automated Email and SMS notifications.

- Designing OLED-based live status display.

- Integrating buzzer-based warning mechanisms.

- Performing hardware assembly and circuit testing.

- Debugging hardware and software issues.

- Evaluating system performance under different environmental conditions.

- Preparing technical documentation, presentation slides, and the internship report.

The project was completed collaboratively, with all team members contributing equally to planning, implementation, testing, and documentation.

2.4 Expected Deliverables

The internship required participants to complete several theoretical, practical, and project-based deliverables.

The expected outcomes included:

Training Deliverables

- Completion of IoT laboratory exercises.

- Understanding of embedded systems and sensor interfacing.

- Development of ESP32-based applications.

- Practical knowledge of cloud communication platforms.

- Experience in hardware-software integration.

Project Deliverables

- Problem identification and literature review.

- System architecture design.

- Hardware circuit development.

- Embedded software implementation.

- AI/ML-based freshness prediction model.

- Cloud dashboard using ThingSpeak.

- Automated Email and SMS notification system through IFTTT.

- OLED-based live monitoring interface.

- Functional IoT prototype.

- Technical documentation and project presentation.

The successful completion of these deliverables demonstrated the ability to apply classroom knowledge to develop an intelligent engineering solution addressing a real-world problem.

2.5 Weekly Timeline

| Week | Period | Activities |
| --- | --- | --- |
| Week 1 | 10–16 June 2026 | Orientation programme, introduction to IoT, embedded systems, ESP32, sensors, actuators, and Arduino programming. |
| Week 2 | 17–23 June 2026 | Sensor interfacing using DHT11, MQ-2, and LDR; ESP32 programming; hardware experimentation; cloud communication fundamentals. |
| Week 3 | 24–30 June 2026 | Requirement analysis, literature review on food spoilage, project selection, system architecture design, and component selection. |
| Week 4 | 1–7 July 2026 | Hardware assembly, ESP32 programming, ThingSpeak integration, OLED display implementation, and AI/ML model development. |
| Week 5 | 8–14 July 2026 | Sensor calibration, AI/ML integration, IFTTT notification setup, system testing, debugging, and prototype optimization. |
| Week 6 | 15–20 July 2026 | Final testing, validation, performance evaluation, documentation, report preparation, and project presentation. |

2.6 Project Summary

The primary outcome of the internship was the successful development of FreshGuard: IoT-Based Food Freshness Monitoring and Prediction System, an intelligent IoT solution designed to monitor environmental conditions affecting food quality.

The system continuously collects temperature, humidity, gas concentration, and ambient light data using multiple sensors connected to an ESP32 microcontroller. The collected information is transmitted to the ThingSpeak cloud platform, where it is stored and visualized for remote monitoring.

An AI/ML-based prediction model analyzes the sensor data to estimate the freshness level of stored food. When abnormal environmental conditions indicating possible spoilage are detected, the system activates a buzzer, displays warning messages on an OLED screen, and automatically sends Email and SMS alerts through IFTTT.

The project successfully demonstrated the integration of IoT, embedded systems, cloud computing, and Artificial Intelligence into a practical food safety application while providing valuable experience in engineering design, system integration, and collaborative project development.

CHAPTER 3

TRAINING AND LEARNING PHASE

3.1 Orientation Programme

The Summer School '26 Internship Programme commenced with an orientation session at the Indian Institute of Technology Jammu. The orientation familiarized participants with the internship schedule, learning objectives, laboratory facilities, safety guidelines, project expectations, assessment methodology, and report submission process.

The programme aimed to bridge the gap between theoretical engineering concepts and practical implementation by emphasizing project-based learning. Students were encouraged to identify real-world problems and develop innovative engineering solutions using Internet of Things (IoT) technologies.

During the orientation, participants were introduced to the broad applications of IoT in various domains, including:

- Smart Agriculture

- Smart Healthcare

- Industrial Automation

- Environmental Monitoring

- Smart Cities

- Energy Management

- Food Safety and Quality Monitoring

- Transportation Systems

The orientation also emphasized teamwork, engineering ethics, systematic documentation, and effective technical presentation, which were essential components of the internship programme.

3.2 Internet of Things (IoT) Fundamentals

The initial phase of the internship focused on understanding the fundamental concepts of the Internet of Things (IoT).

IoT refers to a network of interconnected physical devices capable of sensing environmental conditions, processing information, communicating through the Internet, and performing intelligent actions without continuous human intervention.

A typical IoT system consists of the following major components:

- Sensors

- Embedded Controller

- Communication Module

- Cloud Platform

- Data Processing Unit

- User Interface

- Actuators or Notification System

The data flow of an IoT system can be represented as:

Environment

↓

Sensors

↓

ESP32 Controller

↓

Data Processing

↓

ThingSpeak Cloud

↓

AI/ML Prediction

↓

Dashboard + Alerts + OLED Display

The internship explained how each component contributes to the complete IoT ecosystem and how intelligent monitoring systems are developed using these interconnected technologies.

3.3 Sensors and Data Acquisition

Sensors are the primary input devices in an IoT system. They continuously monitor physical parameters and convert them into electrical signals that can be processed by a microcontroller.

For the FreshGuard project, the following sensors were studied and implemented.

3.3.1 DHT11 Temperature and Humidity Sensor

The DHT11 sensor measures ambient temperature and relative humidity.

Temperature and humidity significantly affect the shelf life of food products. Elevated temperatures accelerate bacterial growth, while excessive humidity promotes fungal contamination and spoilage.

The DHT11 sensor periodically measures:

- Temperature (°C)

- Relative Humidity (%)

The measured values are transmitted digitally to the ESP32 for further processing and cloud storage.

3.3.2 MQ-2 Gas Sensor

The MQ-2 sensor detects combustible gases and smoke.

During food decomposition, gases such as methane and other volatile organic compounds are released. Although the MQ-2 is not a dedicated food-quality sensor, it effectively detects increasing concentrations of spoilage-related gases, making it suitable for educational food monitoring applications.

The sensor continuously measures gas concentration and provides analog output proportional to gas intensity.

The ESP32 reads these values and determines whether spoilage conditions are developing.

3.3.3 LDR (Light Dependent Resistor)

The Light Dependent Resistor measures ambient light intensity.

Certain food items require storage under controlled lighting conditions. Continuous exposure to light can reduce food quality and nutritional value.

The LDR changes its resistance according to the surrounding light intensity.

The ESP32 converts this analog signal into digital values for monitoring environmental conditions.

3.4 ESP32 Microcontroller and Embedded Programming

The ESP32 served as the central processing unit of the FreshGuard system.

The ESP32 was selected because it provides:

- Built-in Wi-Fi

- High processing speed

- Multiple GPIO pins

- Analog and digital input support

- Low power consumption

- Easy cloud connectivity

Programming was carried out using Arduino IDE.

The major programming concepts learned during the internship included:

- Digital Input/Output

- Analog Input Reading

- Sensor Interfacing

- Functions

- Conditional Statements

- Loops

- Wi-Fi Communication

- OLED Display Programming

- Serial Communication

- Data Upload to ThingSpeak

- Error Handling

- Program Debugging

These concepts formed the foundation for developing the FreshGuard prototype.

3.5 Cloud Communication using ThingSpeak

One of the most important learning outcomes of the internship was understanding cloud-based IoT communication.

ThingSpeak is an open-source IoT cloud platform used for:

- Data Collection

- Remote Monitoring

- Data Visualization

- Graph Generation

- Analytics

The ESP32 periodically uploads sensor readings to ThingSpeak using Wi-Fi.

The uploaded parameters include:

- Temperature

- Humidity

- Gas Concentration

- Light Intensity

ThingSpeak stores these readings in separate cloud fields and automatically generates real-time graphs.

Cloud storage enables users to monitor food storage conditions remotely from any Internet-connected device.

The platform also supports historical data analysis, making it useful for identifying trends in environmental conditions over time.

3.6 AI/ML-Based Freshness Prediction

A major highlight of the FreshGuard project was the integration of Artificial Intelligence and Machine Learning.

Traditional monitoring systems only display sensor values.

FreshGuard goes one step further by predicting food freshness using environmental data.

During the internship, participants learned the basic workflow of machine learning:

- Data Collection

- Data Cleaning

- Feature Selection

- Model Training

- Model Testing

- Prediction

- Decision Making

The AI model uses:

- Temperature

- Humidity

- Gas Concentration

- Ambient Light

to estimate the freshness level of stored food.

The predicted freshness status is categorized into:

- Fresh

- Moderate

- Spoilage Warning

- Spoiled

This intelligent prediction enables users to take preventive action before food becomes unsafe.

3.7 Automated Notification using IFTTT

The internship also introduced cloud automation through IFTTT (If This Then That).

IFTTT connects ThingSpeak with communication services.

Whenever the uploaded sensor values exceed predefined thresholds, ThingSpeak triggers an event that activates IFTTT.

IFTTT automatically sends:

- Email Notifications

- SMS Alerts

to inform users about possible food spoilage.

This feature allows remote monitoring even when the user is away from the storage location.

3.8 OLED Display and Local Monitoring

Besides cloud monitoring, FreshGuard also provides local visualization using an OLED display.

The display continuously presents:

- Temperature

- Humidity

- Gas Level

- Freshness Status

- Warning Messages

This enables users to check food conditions directly without requiring Internet access.

3.9 Tools and Equipment Used

The following hardware and software tools were used during the internship.

Hardware

- ESP32 Development Board

- DHT11 Sensor

- MQ-2 Gas Sensor

- LDR

- OLED Display

- Buzzer

- Breadboard

- Jumper Wires

- USB Cable

- Power Supply

Software

- Arduino IDE

- ThingSpeak

- IFTTT

- Serial Monitor

- GitHub (Documentation)

- Microsoft PowerPoint

- Microsoft Word

3.10 Drone Technology Training

During the final phase of the internship, participants received an introductory session on drone technology.

The training covered:

- Drone Architecture

- Brushless DC Motors

- Electronic Speed Controllers (ESC)

- Flight Controllers

- Propellers

- Batteries

- GPS Systems

- Wireless Communication

- Flight Dynamics

- PID Control

- Drone Safety

- Industrial Applications of Drones

Although drone technology was independent of the FreshGuard project, the sessions provided valuable insight into another important application of embedded systems, wireless communication, and real-time control.

3.11 Learning from the Training Phase

The training phase provided a comprehensive understanding of modern IoT technologies and intelligent embedded systems.

The knowledge gained during this period directly contributed to the successful development of FreshGuard.

The training improved practical skills in:

- Embedded Programming

- ESP32 Development

- Sensor Interfacing

- Cloud Computing

- Artificial Intelligence

- Hardware Integration

- IoT Communication

- Real-Time Monitoring

- System Testing

- Technical Documentation

The concepts learned during the training formed the foundation for designing, implementing, testing, and validating the FreshGuard prototype.

CHAPTER 4

PROJECT WORK

4.1 Project Introduction

The major project undertaken during the Summer School '26 Internship Programme was FreshGuard: IoT-Based Food Freshness Monitoring and Prediction System.

FreshGuard is an intelligent IoT-enabled system developed to continuously monitor environmental conditions affecting food quality and predict food freshness using Artificial Intelligence and Machine Learning techniques. The project integrates embedded hardware, environmental sensing, cloud computing, and predictive analytics into a single platform capable of assisting users in preserving food quality and reducing food wastage.

The system utilizes an ESP32 microcontroller connected to multiple sensors, including the DHT11 for temperature and humidity measurement, the MQ-2 gas sensor for detecting spoilage-related gases, and an LDR for measuring ambient light intensity. These parameters are continuously monitored because they directly influence the shelf life and freshness of stored food.

The collected sensor readings are processed by the ESP32 and transmitted via Wi-Fi to the ThingSpeak cloud platform, where they are stored, visualized, and analyzed. An AI/ML-based prediction model estimates the freshness level of food based on environmental conditions and historical sensor data. Whenever abnormal conditions are detected, the system activates a buzzer, displays warning messages on an OLED display, and generates Email/SMS notifications using IFTTT.

The project demonstrates the practical application of IoT, embedded systems, cloud computing, and Artificial Intelligence in smart food quality management.

4.2 Problem Statement

Food spoilage is a significant challenge affecting households, restaurants, supermarkets, food processing industries, and supply chains. Large quantities of food are discarded each year because spoilage is detected only after visible deterioration or unpleasant odor develops.

Traditional methods of assessing food freshness rely mainly on:

- Visual inspection

- Smell

- Texture

- Expiry dates

These methods are subjective, inconsistent, and incapable of providing continuous monitoring.

Environmental factors such as:

- Temperature

- Humidity

- Gas concentration

- Storage conditions

directly influence food deterioration.

Without continuous monitoring, users cannot detect early spoilage, resulting in:

- Food wastage

- Economic losses

- Health hazards

- Reduced food quality

Therefore, there is a need for an intelligent monitoring system capable of continuously observing storage conditions, predicting freshness, and providing timely alerts before food becomes unsafe for consumption.

FreshGuard was developed to address this problem by integrating IoT technology with Artificial Intelligence for real-time food freshness monitoring and prediction.

4.3 Project Objectives

The primary objectives of FreshGuard are:

- To continuously monitor environmental conditions affecting food freshness.

- To measure temperature and humidity using the DHT11 sensor.

- To detect spoilage-related gases using the MQ-2 sensor.

- To monitor ambient light intensity using an LDR.

- To collect and process sensor data using an ESP32 microcontroller.

- To upload real-time sensor readings to the ThingSpeak cloud platform.

- To develop an AI/ML model capable of predicting food freshness.

- To classify food into Fresh, Moderate, Warning, or Spoiled categories.

- To display sensor readings and freshness status on an OLED display.

- To activate a buzzer whenever unsafe conditions are detected.

- To generate automated Email and SMS notifications through IFTTT.

- To demonstrate an intelligent IoT solution for reducing food wastage and improving food safety.

4.4 Team Contribution

The FreshGuard project was developed collaboratively by a four-member team.

Team Members

- Khushkaranpreet kaur

- Ridhika Bansal

- Sayan Biswas

- Tanish Kumar Aggarwal

Each member actively participated throughout the project lifecycle. Responsibilities were distributed equally to encourage collaborative learning and effective project execution.

Major activities performed jointly included:

- Literature survey

- Problem identification

- Requirement analysis

- Component selection

- Hardware procurement

- Circuit design

- ESP32 programming

- Sensor interfacing

- ThingSpeak integration

- AI/ML model implementation

- IFTTT automation

- Hardware assembly

- Prototype testing

- System debugging

- Documentation

- Presentation preparation

Continuous discussion and collaborative decision-making ensured successful project completion.

4.5 Overall Architecture

| Component | Purpose |
| --- | --- |
| ESP32 | Main controller responsible for data processing and Wi-Fi communication |
| DHT11 | Measures temperature and humidity |
| MQ-2 | Detects gases released during food spoilage |
| LDR | Measures ambient light intensity |
| OLED Display | Displays live sensor readings and freshness status |
| Buzzer | Generates audible alerts during abnormal conditions |
| Breadboard | Circuit assembly |
| Jumper Wires | Electrical connections |
| USB Power Supply | System power |

4.6 System Workflow

Food Storage Environment

│

┌────────────────┼────────────────┐

│                │                │

DHT11            MQ-2             LDR

│                │                │

└────────────────┼────────────────┘

│

ESP32 Controller

│

┌───────────────┼───────────────────┐

│               │                   │

OLED Display     ThingSpeak Cloud      Buzzer

│

AI/ML Prediction

│

IFTTT Email/SMS Alerts

The ESP32 acts as the central controller, collecting sensor data, processing readings, uploading information to the cloud, and controlling local output devices.

4.7 Working Principle

The operation of FreshGuard follows the sequence below:

Step 1

The ESP32 initializes all connected sensors and establishes Wi-Fi communication.

Step 2

The DHT11 measures:

- Temperature

- Humidity

Step 3

The MQ-2 detects spoilage-related gases.

Step 4

The LDR measures ambient light intensity.

Step 5

The ESP32 processes all sensor readings.

Step 6

Sensor values are uploaded to ThingSpeak every predefined interval.

Step 7

The AI/ML prediction model analyzes:

- Temperature

- Humidity

- Gas concentration

- Light intensity

to estimate the freshness level.

Step 8

Freshness status is displayed on the OLED screen.

Step 9

If unsafe conditions are detected:

- Buzzer activates.

- ThingSpeak triggers IFTTT.

- Email and SMS notifications are sent.

Step 10

The monitoring cycle repeats continuously.

4.8 AI/ML-Based Freshness Prediction

Unlike conventional IoT systems that simply display sensor readings, FreshGuard incorporates Artificial Intelligence for predictive decision-making.

The AI model receives four environmental parameters as input:

- Temperature

- Humidity

- Gas concentration

- Light intensity

The prediction workflow includes:

- Data Collection

- Data Cleaning

- Feature Selection

- Model Training

- Prediction

- Freshness Classification

Based on predicted values, food is classified into four categories:

| Freshness Level | Description |
| --- | --- |
| Fresh | Safe for consumption |
| Moderate | Slight quality degradation |
| Warning | Consume soon |
| Spoiled | Unsafe for consumption |

4.9 ThingSpeak Cloud Integration

ThingSpeak serves as the cloud platform responsible for:

- Data storage

- Real-time visualization

- Historical analysis

- Dashboard creation

Four fields were created:

- Field 1 → Temperature

- Field 2 → Humidity

- Field 3 → Gas Level

- Field 4 → Light Intensity

The dashboard generates live graphs that allow users to remotely monitor food storage conditions.

4.10 IFTTT Notification System

To enable remote alerts, ThingSpeak is integrated with IFTTT.

Whenever sensor values exceed predefined thresholds:

ThingSpeak → Trigger Event → IFTTT → Email/SMS

The notification includes:

- Warning message

- Timestamp

- Freshness status

This allows users to take corrective action immediately.

4.11 OLED Display

The OLED screen continuously displays:

- Temperature

- Humidity

- Gas Level

- Light Level

- Freshness Status

- Warning Messages

This enables local monitoring without requiring Internet access.

4.12 System Algorithm:

Start

↓

Initialize ESP32

↓

Initialize Sensors

↓

Connect Wi-Fi

↓

Read DHT11

↓

Read MQ-2

↓

Read LDR

↓

Process Sensor Values

↓

Upload to ThingSpeak

↓

AI Freshness Prediction

↓

Display Results on OLED

↓

Threshold Crossed?

↓

YES → Activate Buzzer

↓

Trigger IFTTT

↓

Send Email/SMS

↓

Repeat

4.13 Prototype Testing

The prototype was tested under various environmental conditions.

The following observations were recorded:

| Test | Expected Result | Outcome |
| --- | --- | --- |
| Temperature Increase | Sensor values updated | Successful |
| Humidity Variation | Accurate readings | Successful |
| Gas Exposure | MQ-2 detected gas | Successful |
| Cloud Upload | ThingSpeak updated | Successful |
| AI Prediction | Freshness predicted | Successful |
| OLED Display | Live monitoring | Successful |
| Email Alert | Notification received | Successful |
| SMS Alert | Notification received | Successful |
| Buzzer | Activated during abnormal condition | Successful |

CHAPTER 5

WEEKLY WORK LOG

| Week | Activities Completed | Key Learning |
| --- | --- | --- |
| Week 1 | Attended orientation programme and studied IoT fundamentals, embedded systems, ESP32 architecture, sensors, and Arduino IDE. | Understood IoT architecture, embedded programming concepts, and applications of smart sensing systems. |
| Week 2 | Interfaced DHT11, MQ-2, and LDR sensors with ESP32. Performed basic sensor calibration and data acquisition. | Learned sensor interfacing, analog and digital signal processing, and environmental data collection. |
| Week 3 | Conducted literature survey on food spoilage monitoring, finalized project requirements, selected hardware components, and prepared the system architecture. | Improved requirement analysis, project planning, component selection, and technical research skills. |
| Week 4 | Developed the hardware prototype, integrated sensors with ESP32, configured ThingSpeak cloud platform, and implemented OLED display functionality. | Learned hardware integration, cloud communication, Wi-Fi connectivity, and real-time monitoring. |
| Week 5 | Developed AI/ML freshness prediction logic, integrated IFTTT for Email/SMS alerts, tested sensor performance, and optimized system operation. | Enhanced understanding of AI integration, cloud automation, debugging, and IoT system optimization. |
| Week 6 | Performed complete system validation, documentation, report preparation, presentation development, and final project demonstration. | Improved technical documentation, teamwork, presentation skills, and project validation techniques. |

CHAPTER 6

CHALLENGES FACED

Developing FreshGuard involved several hardware, software, and integration challenges. These challenges provided valuable practical learning opportunities and improved our problem-solving abilities.

6.1 Sensor Calibration

The MQ-2 gas sensor required sufficient warm-up time before producing stable readings. Initially, sensor values fluctuated significantly, resulting in inconsistent gas concentration measurements.

Resolution:

A warm-up period was incorporated into the initialization routine before collecting sensor data. Multiple sensor readings were averaged to reduce fluctuations and improve measurement stability.

6.2 Environmental Variations

Temperature and humidity values changed frequently due to surrounding environmental conditions, affecting the consistency of AI predictions.

Resolution:

Threshold values were optimized after repeated experimentation. The AI model was designed to analyze multiple environmental parameters simultaneously instead of relying on a single sensor value.

6.3 Wi-Fi Connectivity Issues

Occasional interruptions in Wi-Fi connectivity prevented successful data transmission to the ThingSpeak cloud platform.

Resolution:

Automatic Wi-Fi reconnection logic was implemented within the ESP32 program. Sensor data continued to be collected locally and uploaded once network connectivity was restored.

6.4 Cloud Communication Delays

During testing, cloud updates occasionally experienced delays due to Internet latency and ThingSpeak update intervals.

Resolution:

Appropriate data transmission intervals were configured according to ThingSpeak limitations. The system was also designed to display sensor values locally on the OLED display to ensure uninterrupted monitoring.

6.5 AI Model Optimization

Initially, the freshness prediction model produced inconsistent classifications because of limited training data.

Resolution:

Additional datasets were collected under different environmental conditions. Feature selection was improved by considering gas concentration, temperature, humidity, and light intensity simultaneously, resulting in more reliable predictions.

6.6 Hardware Integration

Integrating multiple sensors with ESP32 while maintaining stable communication required careful wiring and power management.

Resolution:

Each hardware module was tested independently before full system integration. Proper grounding, organized wiring, and modular testing minimized hardware failures.

6.7 Team Coordination

The project involved four team members working simultaneously on hardware development, software programming, AI implementation, cloud integration, and documentation.

Resolution:

Responsibilities were divided among team members while maintaining regular discussions and progress reviews. Continuous communication ensured smooth integration of all project modules.

6.8 Time Management

Completing the project within the six-week internship required balancing technical development, testing, documentation, and presentations.

Resolution:

A weekly development schedule was prepared, with project milestones reviewed regularly. Critical functionalities were prioritized before implementing advanced features.

CHAPTER 7

SKILLS DEVELOPED

7.1 Technical Skills

The internship strengthened practical knowledge in the following technical areas:

- Internet of Things Architecture

- Embedded Systems

- ESP32 Programming

- Arduino IDE

- Sensor Interfacing

- Temperature and Humidity Monitoring

- Gas Detection Systems

- Analog and Digital Signal Processing

- Cloud Computing

- ThingSpeak Platform

- IFTTT Automation

- Artificial Intelligence

- Machine Learning Fundamentals

- Environmental Monitoring

- Hardware Integration

- Wireless Communication

- Real-Time Data Acquisition

- OLED Display Programming

- System Testing and Validation

- Technical Documentation

7.2 Teamwork Skills

Working collaboratively on FreshGuard improved several interpersonal and project management skills.

These included:

- Effective communication

- Collaborative decision making

- Technical discussion

- Task allocation

- Problem solving

- Peer learning

- Conflict resolution

- Documentation sharing

- Presentation preparation

- Project coordination

The experience demonstrated the importance of teamwork in successfully completing multidisciplinary engineering projects.

7.3 Professional Skills

The internship also enhanced several professional competencies essential for engineering careers.

These included:

- Communication Skills

- Time Management

- Critical Thinking

- Technical Report Writing

- Project Planning

- Presentation Skills

- Research Methodology

- Independent Learning

- Creativity

- Leadership

- Engineering Ethics

- Analytical Thinking

- Adaptability

- Attention to Detail

CHAPTER 8

LEARNING OUTCOMES

The Summer School '26 Internship Programme provided valuable practical exposure to modern engineering technologies while reinforcing classroom concepts through real-world implementation.

The internship demonstrated that successful IoT system development requires the integration of multiple disciplines, including embedded systems, cloud computing, wireless communication, data analysis, and Artificial Intelligence.

Throughout the internship, I learned the complete engineering design process, beginning with identifying a practical problem and ending with the successful development of a working prototype.

The major learning outcomes are summarized below.

8.1 Technical Learning

The internship improved my understanding of:

- Internet of Things architecture

- Sensor technologies

- Embedded programming

- ESP32 development

- Hardware-software integration

- Cloud-based monitoring

- ThingSpeak analytics

- Environmental sensing

- Artificial Intelligence

- Machine Learning concepts

- Automated notification systems

- Data visualization

- Real-time monitoring systems

- Wireless communication

I also gained practical experience in developing an end-to-end IoT solution capable of acquiring, processing, analyzing, and transmitting sensor data for intelligent decision-making.

8.2 Project-Based Learning

The FreshGuard project demonstrated how theoretical engineering concepts can be transformed into practical solutions.

Through project implementation, I learned:

- Requirement analysis

- Literature review

- Hardware selection

- Circuit development

- Embedded software design

- Sensor calibration

- AI model integration

- Cloud communication

- Prototype testing

- System validation

- Documentation

- Technical presentation

These experiences significantly strengthened my engineering design abilities.

8.3 Team Learning

Working in a multidisciplinary team improved my ability to:

- Coordinate project activities

- Share responsibilities

- Review technical work

- Solve engineering problems collaboratively

- Respect different viewpoints

- Maintain project documentation

- Deliver presentations as a team

The project reinforced the importance of collaboration in engineering innovation.

8.4 Overall Learning Experience

The internship successfully combined theoretical instruction with practical implementation.

Developing FreshGuard enhanced my confidence in designing intelligent IoT systems capable of solving real-world problems.

The project also demonstrated the potential of combining Internet of Things technologies with Artificial Intelligence for developing smarter, safer, and more sustainable engineering solutions.

The knowledge and experience gained during the internship have strengthened my foundation in embedded systems, cloud computing, and AI-driven IoT applications, preparing me for future academic research and industrial projects.

CHAPTER 9

SELF-ASSESSMENT

9.1 Strengths

The Summer School '26 Internship Programme provided an excellent opportunity to evaluate my technical and professional abilities through practical project development. Working on FreshGuard helped me identify several strengths that contributed to the successful completion of the project.

My major strengths during the internship included:

- Strong willingness to learn new technologies.

- Ability to understand theoretical concepts and apply them to practical problems.

- Interest in Internet of Things, Artificial Intelligence, and embedded systems.

- Good analytical and logical thinking.

- Effective teamwork and collaboration.

- Patience while troubleshooting hardware and software issues.

- Ability to adapt to new development environments.

- Responsibility in completing assigned tasks on time.

- Attention to detail during circuit assembly and testing.

- Confidence in technical documentation and report writing.

These strengths enabled me to actively contribute to the successful implementation of the FreshGuard project while continuously improving my engineering knowledge.

9.2 Areas for Improvement

Although the internship significantly improved my technical skills, it also highlighted several areas where further learning and practical experience are required.

The areas that I intend to improve include:

- Advanced Machine Learning model development.

- Deep Learning applications in IoT.

- Edge AI implementation on embedded devices.

- IoT cybersecurity and secure communication protocols.

- PCB design using professional EDA software.

- Industrial-grade sensor calibration techniques.

- Mobile application development for IoT systems.

- Cloud analytics and dashboard customization.

- Industrial automation protocols.

- Large-scale IoT deployment.

- Advanced embedded system optimization.

- Technical leadership and project management.

Developing expertise in these areas will enable me to build more efficient, secure, and scalable IoT solutions in the future.

9.3 Reflection on the Internship Experience

The Summer School '26 Internship Programme was one of the most valuable learning experiences of my academic journey. It provided practical exposure to modern engineering technologies while allowing me to work on a real-world problem through project-based learning.

Initially, the internship focused on understanding the fundamentals of IoT, embedded systems, sensors, wireless communication, and cloud computing. These concepts formed the foundation for developing FreshGuard.

Working on the project gave me practical experience in:

- Problem identification

- Requirement analysis

- Hardware selection

- Sensor interfacing

- Embedded programming

- Cloud integration

- Artificial Intelligence

- Machine Learning

- Testing

- Documentation

Collaborating with my teammates further improved my communication skills, teamwork, and ability to solve engineering challenges collectively.

One of the most rewarding aspects of the internship was witnessing how multiple technologies—including IoT, AI, cloud computing, and embedded systems—can be integrated to develop an intelligent solution for improving food safety and reducing food wastage.

Overall, the internship strengthened my technical foundation, increased my confidence in engineering design, and motivated me to continue exploring advanced research in Artificial Intelligence and Internet of Things.

CHAPTER 10

CONCLUSION AND FUTURE SCOPE

10.1 Conclusion

The Summer School '26 Internship Programme at the Indian Institute of Technology Jammu successfully provided theoretical knowledge and practical experience in Internet of Things technologies, embedded systems, cloud computing, Artificial Intelligence, and intelligent system development.

The structured learning approach combined classroom instruction, laboratory experimentation, and project-based implementation, allowing participants to apply engineering concepts to real-world applications.

The major achievement of the internship was the successful development of FreshGuard: IoT-Based Food Freshness Monitoring and Prediction System.

The system integrates:

- ESP32 Microcontroller

- DHT11 Temperature and Humidity Sensor

- MQ-2 Gas Sensor

- LDR

- OLED Display

- ThingSpeak Cloud Platform

- Artificial Intelligence and Machine Learning

- IFTTT Email and SMS Notifications

The developed prototype continuously monitors environmental conditions affecting food freshness, predicts spoilage using AI/ML techniques, and generates timely alerts through cloud-based communication.

The project demonstrated the practical implementation of modern IoT technologies for addressing a significant real-world problem.

Besides technical knowledge, the internship strengthened my abilities in:

- Engineering design

- Embedded programming

- Cloud computing

- Artificial Intelligence

- Hardware integration

- Testing

- Technical documentation

- Teamwork

- Communication

- Problem solving

Overall, the internship significantly enhanced both my technical competence and professional confidence, preparing me for future research and industrial applications in IoT and Artificial Intelligence.

10.2 Future Scope

Although FreshGuard successfully demonstrated the feasibility of intelligent food freshness monitoring, several enhancements can further improve the system.

Possible future improvements include:

Hardware Improvements

- Industrial-grade gas sensors for improved accuracy.

- Additional sensors for CO₂, ammonia, and ethylene detection.

- High-precision temperature and humidity sensors.

- Rechargeable battery backup.

- Custom PCB development.

- Compact enclosure for commercial deployment.

Software Improvements

- Dedicated Android and iOS mobile applications.

- Advanced cloud dashboards.

- Multi-user monitoring.

- Real-time analytics.

- Remote firmware updates.

Artificial Intelligence Improvements

- Deep Learning-based freshness prediction.

- Computer Vision for food quality inspection.

- Image-based spoilage detection.

- Self-learning AI models.

- Personalized prediction algorithms.

IoT Improvements

- MQTT communication protocol.

- AWS IoT integration.

- Google Cloud IoT.

- Azure IoT Hub.

- Edge AI implementation.

- Smart inventory management.

Industrial Applications

The FreshGuard system can be extended for use in:

- Supermarkets

- Cold storage warehouses

- Dairy industries

- Food processing plants

- Restaurants

- Hotels

- Hospitals

- Household refrigerators

- Smart kitchens

- Agricultural supply chains

Such improvements would increase the commercial viability and scalability of the system.

10.3 Career Plans

The internship has inspired me to pursue further learning and research in the fields of:

- Internet of Things

- Embedded Systems

- Artificial Intelligence

- Machine Learning

- Data Analytics

- Cloud Computing

- Smart Automation

- Intelligent Monitoring Systems

In the future, I aim to contribute to the development of innovative engineering solutions that improve quality of life through intelligent technologies.

The practical experience gained during this internship has strengthened my motivation to pursue advanced projects, internships, research opportunities, and higher studies in emerging technologies.

APPENDICES

Appendix A: Internship Offer Letter

Attach the internship offer letter.

Appendix B: Internship Completion Certificate

Attach the completion certificate.

Appendix C: Project Photographs

Appendix D: Presentation Video

https://drive.google.com/drive/folders/1FZ0apCmC1mrVvO3x_jBFqnJscC1Eokug