| Feature | Analogread() | Analogwrite() |
| --- | --- | --- |
| Purpose | Reads an external voltage from an analog pin. | Generates a pseudo-analog signal using PWM on specific digital pins |
| Data flow | Input (sensor to arduino) | Output (Arduino to actuator) |
| range | 0 to 1023(10 – bit resolution) | 0 to 255 (8-bit resolution) |
| Example | Reading a potentiometer | Fading an led |

Q2. What is PWM and why it is used?

Ans. PWM (Pulse Width Modulation) is a technique used to simulate an analog output using digital signals. Since a standard digital pin can only be HIGH (5V) or LOW (0V), PWM switches the pin on and off extremely fast. By changing the ratio of time the signal is "ON" versus "OFF" (known as the Duty Cycle), we can control the average voltage delivered to a device.

​. Why use it?

It allows us to control the "intensity" of devices like motors (speed) or LEDs (brightness) without needing expensive dedicated Digital-to-Analog Converter (DAC) hardware.

Q3. Practical example of IOT.

Ans .

​For analogRead() (Sensors): Using an LDR (Light Dependent Resistor) in an IoT smart-lighting project to measure ambient light levels and trigger a relay to turn on street lamps when it gets dark.

​For analogWrite() (Actuators): Using a PWM pin to control the speed of a small cooling fan in a remote-controlled IoT server room monitor, where the fan speed increases as the detected temperature rises.